LINE-2D Shape-Based Matching

Technical Handover Document

Wafer Alignment System — QES (Asia-Pacific) Sdn Bhd

May 2026

# 1. Executive Summary

This document describes the LINE-2D shape-based template matching subsystem used in the Wafer Alignment application. The algorithm locates a trained template pattern in a search image by comparing quantised gradient orientations. It is a Python port of the C++ SBM_line2D implementation (reference: SeanYan604/SBM_line2D).

Key capabilities: rotation-invariant matching (0°–360°, configurable step), multi-scale support, sub-pixel position and angle refinement, and coarse-to-fine pyramid acceleration for high-resolution (>2000 px) images.

# 2. Architecture Overview

## 2.1 MVVM Layer Mapping

| Layer | File | Role |
| --- | --- | --- |
| Service | linemod_matcher.py | Core algorithm (1330 lines) |
| ViewModel | pattern_viewmodel.py | UI logic, config, caching |
| View | pattern_tab.py | Tkinter widgets |
| Model | recipe_model.py | XML recipe persistence |
| Model | app_state.py | Shared mutable state |
| Integration | zmq_server.py | ZeroMQ TCP interface |

## 2.2 Dependency Flow

pattern_tab (View) → PatternViewModel → LinemodMatcher (Service)
PatternViewModel → RecipeManager (Model)
ZMQ Server → LinemodMatcher + RecipeManager

# 3. Algorithm Pipeline

The matching pipeline has two phases: offline (template generation) and online (search-image matching).

## 3.1 Phase 1 — Template Generation (generate_templates)

Rotation & Scale Grid: For each (angle, scale) pair the base template image is rotated and scaled via cv2.warpAffine. Default: 5° step × 1 scale = 72 templates.

Mask Handling: A rotation mask (all-white image transformed identically) prevents border artefacts. An optional user-drawn detection mask restricts feature extraction.

Pyramid Construction: For each pyramid level l ∈ {0,1,2}, the image is downsampled by 2^l via cv2.pyrDown.

Gradient Quantisation: At each level, Sobel gradients → angle → 8 orientation bins.

Feature Extraction: N spatially-scattered features selected by gradient magnitude.

Bounding-Box Crop: _crop_templates computes the tightest bounding box and makes feature coordinates relative.

NumPy Pre-caching: Feature (x,y,label) arrays cached as int32 vectors for inner-loop speed.

Template generation is parallelised with ThreadPoolExecutor (up to 8 workers). OpenCV/NumPy release the GIL so threads run in parallel.

## 3.2 Phase 2 — Online Matching (match)

Two execution paths exist, selected automatically by image size:

Single-Level (≤ 2000 px): Full-resolution scan.

Pyramid (> 2000 px): Coarse-to-fine search.

# 4. Core Algorithms

## 4.1 Gradient Quantisation

Function: _quantize_gradients(src, weak_threshold, fast_mode, kernel_size)

Convert to float32 grayscale.

Optional Gaussian blur (auto-sigma for images >1500 px, or explicit kernel).

Sobel derivatives: dx = Sobel(I, x), dy = Sobel(I, y), ksize=3.

Magnitude: M = sqrt(dx² + dy²); Angle: θ = atan2(dy, dx) mod 360°.

Quantise to 16 bins then fold to 8: quant_8 = (angle × 16/360).astype(uint8) & 7.

Adaptive threshold: If weak_threshold < 0, use the |value|-th percentile of non-zero magnitudes.

### Hysteresis Mode (fast_mode=False)

For each of 8 orientation bins, count votes in a ks×ks neighbourhood via cv2.filter2D. Accept a pixel only if M > threshold AND the winning bin has ≥ ⌈ks²/2⌉ + 1 votes. Kernel size: auto = min(max(3, ⌊3 × maxdim/1500⌋) | 1, 9).

### Fast Mode (fast_mode=True)

Skip hysteresis; accept any pixel where M > threshold. Used for search images where speed matters.

Output: A uint8 image where each pixel is a power-of-2 bit (2^bin) or 0.

## 4.2 Orientation Spreading

Function: _spread(quantized, T)

OR-dilates each of the 8 bit-planes independently with a (2T−1) × (2T−1) rectangular kernel via cv2.dilate, then ORs the results. This creates tolerance to small spatial misalignment.

Default T values per pyramid level: [4, 8, 16].

## 4.3 Response Map LUTs

Function: _build_response_luts() → 8 tables of 256 entries.

For each template label L ∈ {0..7} and each possible spread byte v ∈ {0..255}:

LUT[L][v] = 4  if bit L is set in v (exact match)

LUT[L][v] = 1  if a neighbour bit (L±1 mod 8) is set (neighbour match)

LUT[L][v] = 0  otherwise

Response maps are computed via cv2.LUT(spread_img, LUT[label]) — a single vectorised lookup per label.

## 4.4 Scattered Feature Selection

Function: _extract_scattered_features(quantized, magnitude, N, mask)

Collect all non-zero quantised pixels (optionally masked).

Sort by descending gradient magnitude.

Greedily select N features with minimum-distance constraint. Distance starts at d = perimeter / (0.8N) and shrinks by 15% per pass.

Uses spatial grid hashing: distance checks are O(9 × k) per candidate instead of O(n).

## 4.5 Score Map Computation

For each template, accumulate response values at feature locations:

S(r,c) = Σ ResponseMap[label_i][r + y_i, c + x_i]  for i=1..N_feat

similarity(r,c) = 100 × S(r,c) / (4 × N_valid)

Features are sorted by label to improve L2 cache hit rate (same response map accessed consecutively). Score maps use int16 dtype (max 128 × 4 = 512 < 32767).

## 4.6 Sub-Pixel Position Refinement

Function: _subpixel_refine(score_map, r, c)

3-point parabolic fit on each axis independently:

δx = (S[r,c-1] - S[r,c+1]) / (2 × (S[r,c-1] - 2×S[r,c] + S[r,c+1]))

Offset clamped to [-0.5, +0.5].

## 4.7 Angular Sub-Pixel Interpolation

Function: _angular_interpolate(matches)

Given the best discrete match at angle θ₀ with score S₀, and scores S₋, S₊ from the adjacent angle templates:

δθ = (Δθ/2) × (S₋ - S₊) / (S₋ - 2×S₀ + S₊),  |δθ| ≤ Δθ/2

## 4.8 Non-Maximum Suppression

Function: _nms(matches)

Sort by score descending. Keep a match only if its Euclidean distance to all previously kept matches exceeds NMS_DISTANCE (default 30 px).

# 5. Pyramid Matching

Function: _match_pyramid(search_gray, threshold)

## 5.1 Coarse Pass (Level N)

Downsample search image by 2^N via cv2.pyrDown. Level N = min(PYRAMID_LEVELS−1, ⌈log₂(maxdim/1200)⌉).

Quantise, spread (with T_N), compute response maps at coarse resolution.

Score each template using only the first COARSE_NUM_FEATURES (default 32) features.

Lower threshold: max(0.3 × threshold, 15).

Keep top candidate per template. Cap total to MAX_FINE_CANDIDATES.

## 5.2 Fine Pass (Level 0)

Cluster overlapping ROI windows into super-ROIs.

For each super-ROI: quantise (fast mode), spread, build response maps once.

Score each candidate template within its sub-window of the super-ROI.

Full feature set (N=128) at full resolution for precise localisation.

# 6. Configuration Reference

| Parameter | Default | Description |
| --- | --- | --- |
| ANGLE_STEP | 5 | Degrees between rotation templates |
| SCALE_MIN/MAX | 1.0/1.0 | Scale range (0.8–1.2 for Full Search) |
| SCALE_STEP | 0.1 | Scale increment |
| WEAK_THRESHOLD | -70 | Gradient threshold. Negative = percentile-adaptive |
| NUM_FEATURES | 128 | Features per template level |
| HYSTERESIS_KERNEL | 0 | Majority-vote kernel size (0=auto) |
| T_PYRAMID | [4,8,16] | Spread T per pyramid level |
| MATCH_THRESHOLD | 50.0 | Minimum similarity (0–100) |
| NMS_DISTANCE | 30 | Non-max suppression radius (px) |
| FAST_SEARCH_QUANTIZE | True | Skip hysteresis on search images |
| COARSE_NUM_FEATURES | 32 | Features for coarse pass |
| MAX_COARSE_CANDIDATES | 2 | Top candidates per template (coarse) |
| MAX_FINE_CANDIDATES | 5 | Cap before fine pass |

# 7. Search Modes (UI)

| Mode | ANGLE_STEP | Scale | Templates |
| --- | --- | --- | --- |
| Simple (Fast) | 360 (only 0°) | 1.0 | 1 |
| With Rotation | 10 | 1.0 | 36 |
| Full Search | 10 | 0.8–1.2 | ~180 |

# 8. Data Structures

## 8.1 Feature

class Feature:
    __slots__ = ['x', 'y', 'label']  # x,y: position; label: orientation bin 0-7

## 8.2 TemplatePyr

class TemplatePyr:
    __slots__ = ['width', 'height', 'tl_x', 'tl_y',
                 'pyramid_level', 'features',
                 'feat_xs_arr', 'feat_ys_arr', 'feat_labels_arr']

## 8.3 Template Pyramid Entry (dict)

{ 'angle': float, 'scale': float,
  'templates': [TemplatePyr, ...],  # one per pyramid level
  'size': (width, height) }

## 8.4 Match Result (dict)

{ 'x': float, 'y': float,          # sub-pixel centre
  'angle': float,                   # refined angle (degrees)
  'angle_discrete': float,          # original grid angle
  'angle_interp_delta': float,      # interpolation offset
  'scale': float, 'score': float,   # 0-100 similarity
  'template_id': int,
  'bbox': (x, y, w, h) }

# 9. ZMQ Integration

## 9.1 PM_REQ Command

PM_REQ "<image_path>" "<recipe_path>"

Response (JSON):

{"status":"ok", "x":..., "y":..., "angle":..., "score":..., "delta_x":..., "delta_y":...}

## 9.2 TEACH_REQ Command

TEACH_REQ "<image_path>" "<recipe_path>"

Opens interactive OpenCV ROI selector. Operator crops template, optionally draws detection mask. Saves to recipe XML.

# 10. Recipe XML Schema (FindPattern)

<FindPattern>
  <MatchThreshold>50.0</MatchThreshold>
  <NumFeatures>128</NumFeatures>
  <GradThrPct>70.0</GradThrPct>
  <TSpread>4</TSpread>
  <HystKernel>0</HystKernel>
  <SearchMode>Simple (Fast)</SearchMode>
  <TemplatePath>C:\...\template.png</TemplatePath>
  <TemplateCropCX>512.0</TemplateCropCX>
  <TemplateCropCY>310.0</TemplateCropCY>
  <DetectionMaskPath></DetectionMaskPath>
</FindPattern>

# 11. Performance Characteristics

| Phase | 919px | 2560px | 5120px |
| --- | --- | --- | --- |
| Quantise (fast) | 5 ms | 15 ms | 50 ms |
| Spread (T=4) | 8 ms | 25 ms | 90 ms |
| Response Maps | 3 ms | 10 ms | 35 ms |
| Scoring (1 tmpl) | 10 ms | 80 ms | 300 ms |
| Scoring (72 tmpl) | 200 ms | — | — |
| Pyramid total | — | 180 ms | 400 ms |

Memory: Response maps use int16 (halves bandwidth vs int32). Rotated template images are NOT stored; regenerated on-demand for visualisation.

# 12. Detection Mask System

Users can draw a polygon mask on the template to exclude regions (e.g. wafer boundary curvature).

mask_editor.py: OpenCV interactive polygon drawing window with auto-detection of wafer boundary.

Mask stored as <template>_mask.png alongside the template.

Passed to _extract_scattered_features as a binary mask: features only extracted where mask > 0.

Mask is rotated/scaled identically to the template during generate_templates.

# 13. Smart Template Caching

The PatternViewModel tracks a config fingerprint string:

f"{NUM_FEATURES}_{WEAK_THRESHOLD}_{T_PYRAMID}_{HYSTERESIS_KERNEL}_{ANGLE_STEP}_{SCALE_MIN}_{SCALE_MAX}_{img_hash}_{mask_hash}"

Templates are only regenerated when this string changes. This avoids redundant ~2 s rebuilds during slider tuning.

# 14. Known Limitations & Future Work

No GPU acceleration (CUDA Sobel/dilate was prototyped but PCIe overhead exceeded gains for template-sized crops).

Scale search is coarse (0.1 step); finer steps increase template count linearly.

Single-level fallback (_match_single_level) is slow for images >2000 px; pyramid path should always be preferred.

No rotation-invariant NMS (matches at different angles near the same position may both survive).

# 15. File Reference

| File | Purpose |
| --- | --- |
| app/services/linemod_matcher.py | Core LINE-2D algorithm (1330 lines) |
| app/viewmodels/pattern_viewmodel.py | UI business logic, config, timing chart |
| app/views/pattern_tab.py | Tkinter tab UI (sliders, buttons, image display) |
| app/views/mask_editor.py | Interactive polygon mask editor |
| app/models/recipe_model.py | XML recipe read/write |
| app/models/app_state.py | Shared state (template crop centre, mask, flags) |
| app/services/zmq_server.py | ZMQ server (PM_REQ, TEACH_REQ handlers) |